TRANSFER FUNCTION

%For input = Unit Impulse function

syms s t;

Num = [4,1];

Den = [4,4,1];

Transfer = tf(Num, Den)

Tfun = (4*s+1)/(4*s^2+ 4*s +1);

TRes(t) = ilaplace(Tfun)

t1 = [0:0.1:100];

subplot(1,2,1)

plot(t1,TRes(t1))

%For input = Unit step function

syms s t;

Num = [4,1];

Den = [4,4,1,0];

Transfer = tf(Num, Den)

Tfun = (4*s+1)/(4*s^3+ 4*s^2 +1*s);

TRes(t) = ilaplace(Tfun)

t1 = [0:0.1:100];

subplot(1,2,2)

plot(t1,TRes(t1))

Experiment – 1

Aim –

To study the time response of type 0, 1 and 2 systems.

Que 1, Obtaining the step response & impulse of a type ‘0’ system

Code –

% Step & impulse response of type '0'

% G(s) = 1/ s^2 +s +4

num=[1];

den=[1 1 4];

g=tf(num,den);

t=feedback(g,1);

subplot(1,2,1);

step(t,'r');

subplot(1,2,2);

impulse(t,'r');

Output –

Que 2, Obtaining the step response & impulse of a type ‘1’ system

Code –

% Step response & impulse response of type '1'

% G(s) = 1/ s(s^2 +s +4)

num=[1];

den=[1 1 4 0];

g=tf(num,den);

t=feedback(g,1);

subplot(1,2,1);

step(t,'r');

subplot(1,2,2);

impulse(t,'g');

Output –

Que 3, Obtaining the step response & impulse of a type ‘2’ system

Code –

% Step response & impulse response of type '2'

% G(s) = 1/ s^2(s^2 +s +4)

num=[1];

den=[1 1 4 0 0];

g=tf(num,den);

t=feedback(g,1);

subplot(1,2,1);

step(t,'r');

subplot(1,2,2);

impulse(t,'r');

Output –

Que 4

% USe of the command stepinfo(sys)

num=[16];

den=[1 1.6 16]

sys=tf(num,den);

S=stepinfo(sys)

load ltiexamples

ltiview

output –

Experiment – 2

Aim –

To study the frequency response of 1st order and 2nd order system.

Apparatus –

MATLAB Software.

Code-

- Pole zero diagram.

% pole zeroes diagram

num=[1 4 3];

den=[1 9 27 35];

g=tf(num,den)

[z,p,k]=tf2zp(num,den)

pzmap(g)

output –

polezeroplot -

g =

s^2 + 4 s + 3

-----------------------

s^3 + 9 s^2 + 27 s + 35

Continuous-time transfer function.

z =

-3

-1

p =

-5.0000 + 0.0000i

-2.0000 + 1.7321i

-2.0000 - 1.7321i

k =

1

Output –

Aim –

To Plot Pole-Zero diagram.

Code –

% Program for Pole-zero configuration in s-plane

% Set up for zeros

z=[j;-j]

%Set up vector for poles

p=[-1;.5+.5j;.5-.5j];

figure(1);

zplane(z,p);

title('Pole/Zeros Plot')

Output –

polezeroplot

z = 0.0000 + 1.0000i

0.0000 - 1.0000i

Graph –

Code –

% to obtain transfer function

p=[-2 1];

z=[-1];

k=7;

[num,den]=zp2tf(z',p',k);

g=tf(num,den)

Output –

>> tfpzd

g =

7 s + 7

-----------

s^2 + s - 2

Continuous-time transfer function.

Aim –

To find the bode plotter.

Code –

% Program to find bode plot

clc

num=input('Enter the value of numerator:')

den=input('Enter the value of denominator:')

fun=tf(num,den)

bode(fun)

grid on

Output –

Enter the value of numerator:[1 4]

num =

1     4

Enter the value of denominator:[4 6 8]

den =

4     6     8

fun =

s + 4

---------------

4 s^2 + 6 s + 8

Continuous-time transfer function.

Graph –

Aim –

To find transfer function and plot pz map.

Code –

% Program to plot pole zero configuration of transfer function

% h(s) = 2s^2 + 5s + 1 / s^2 + 3s +5 ;

n=[2 5 1];

d=[1 3 5];

sys=tf(n,d)

pzmap(sys)

Output –  tfmap

sys =

2 s^2 + 5 s + 1

---------------

s^2 + 3 s + 5

Continuous-time transfer function.

Aim –

To find transfer function and plot the graph.

Code –

% Program to plot pole zero configuration of tranfer function

clc;

clear all;

n1=input('Enter the numerator : ');

d1=input('Enter the denominator : ');

sys=tf(n1,d1)

pzmap(sys)

Output –

1. Enter the numerator : [1]

Enter the denominator : [1 1]

sys =

1

--------

s + 1

Continuous-time transfer function.

Graph –

2. Enter the numerator : [1]

Enter the denominator : [1 10]

sys =

1

-----

s + 10

Continuous-time transfer function.

Graph –

3.

Enter the numerator : [1 1]

Enter the denominator : [1 4 4]

sys =

( s + 1) / (s^2 + 4 s + 4)

Continuous-time transfer function.

Graph –

Experiment – 3

Aim –

To study the time response (rise time, delay time, %maximum overshoot, settling time) of second order control system for step response.

Code –

% Program for unit step response to find rise time and delay time

clc

n=input('Enter the numerator :');

d=input('Enter the denum :');

sys=tf(n,d)

S=stepinfo(sys,'RiseTimeLimits',[0.05,0.95])

step(sys)

grid on

Output –

- H(s)=(2s/s^2+6s+25)

Enter the numerator :[2]

Enter the denum :[1 6 25]

sys =

2

--------------

s^2 + 6 s + 25

Continuous-time transfer function.

S =

RiseTime: 0.4400

SettlingTime: 1.1887

SettlingMin: 0.0774

SettlingMax: 0.0876

Overshoot: 9.4773

Undershoot: 0

Peak: 0.0876

PeakTime: 0.7829

Graph –

2.

H(s)= (1/ s^2 +0.2s+1)

Enter the numerator :[1]

Enter the denum :[1 0.2 1]

sys =

1 / s^2 + 0.2 s + 1

Continuous-time transfer function.

S = RiseTime: 1.3030

SettlingTime: 38.3730

SettlingMin: 0.4685

SettlingMax: 1.7292

Overshoot: 72.9156

Undershoot: 0

Peak: 1.7292

PeakTime: 3.1416

Graph –

3.

h(s) = 2s + 1 / s^2 + s +1;

Enter the numerator :[2 1]

Enter the denum :[1 1 1]

sys =

2 s + 1 /  s^2 + s + 1

Continuous-time transfer function.

S = RiseTime: 0.5412

SettlingTime: 7.3836

SettlingMin: 0.8860

SettlingMax: 1.6991

Overshoot: 69.9080

Undershoot: 0

Peak: 1.6991

PeakTime: 1.8421

Graph –

Aim –

To find rise time and delay time for unit step response.

Code –

% Program for unit step response to find rise time and delay time

clc

sys=tf([8 18 32],[1 6 14 24])

S=stepinfo(sys,'RiseTimelimits',[0.05,0.95])

step(sys)

grid on

Output –

sys =

8 s^2 + 18 s + 32

-----------------------

s^3 + 6 s^2 + 14 s + 24

Continuous-time transfer function.

S =

RiseTime: 0.2393

SettlingTime: 3.4972

SettlingMin: 1.1956

SettlingMax: 1.6871

Overshoot: 26.5302

Undershoot: 0

Peak: 1.6871

PeakTime: 0.5987

Graph –

Experiment – 4

Aim –

Study of P, PI, PD and PID Controller.

Apparatus Required –

MATLAB Software

Theory –

When the plant’s performance isn’t as per the system’s requirements, a closed loop control system can be handy, instead of a new plant. The error between the actual and the required outputs derived the controller. This controller in turn produces the necessary correcting actions upon the plant pulling the actual output towards the required one. The easiest approach is to produce the actuating signal u proportional to the error, the more is the error, the higher will be the actuating signal magnitude. This allows the actual output to follow the required output. This is the Proportional controller. This controller is useful when the error is under some limit.

# PD Controller

Practically, no single controller can serve the purpose. It needs multiple controllers work together to meet the requirements. Whenever, it’s needed to speed up the transient period, we add proportional control and the overshoot and the oscillatory problems need the derivative control. This calls for Proportional- Derivative controller (PD Controller). Special care must be taken to ensure that the signals are noiseless, else the noise can get amplified by the derivative actions and creates stability problems to the plant.

# PI Controller

This helps in the situation where proportional control is necessary to speed up the settling and integral control to reduce the error that is constant over time. This is Proportional Integral controller (PI controller).

# PID Controller

More general case would be to address all the above discussed problems with a single solution. In this case all the three controllers are used in parallel will appropriately chosen gains. Adjusting these gains, any combination of P,I,D controllers can be obtained and hence a more robust one.

The three gains — Kp, Ki and Kd for the Proportional, Integral and Derivative controller actions can decide the contribution of each one in the final controller action.

Code –

% Open loop step reponse

num = 1;

den = [1 10 20];

plant = tf(num,den);

step(plant)

title('open loop step response')

% Proportional control

Kp= 300;

contr = Kp;

sys_cl = feedback(contr*plant,1);

t=0:0.01:2;

step(sys_cl,t)

title('Poportional Control')

% Proportional-Derivative control

Kp=300;

Kd=10;

contr=tf([Kd Kp],1);

sys_cl=feedback(contr*plant,1);

t=0:0.01:2;

step(sys_cl,t)

title('Poportional-Derivative Control')

% Proportional-Integral control

Kp=30;

Ki=70;

contr=tf([Ki Kp],[1 0]);

sys_cl=feedback(contr*plant,1);

t=0:0.01:2;

step(sys_cl,t)

title('Proportional-Integral control')

% Proportional-Integral-Derivative control

Kp=350;

Ki=300;

Kd=50;

contr=tf([Kd Kp Ki],[1 0]);

sys_cl=feedback(contr*plant,1);

t=0:0.01:2;

step(sys_cl,t)

title('Proportional-Integral-Derivative control')

Output –

Result –

We have successfully drawn the characteristics of P, PI, PD and PID Controller.